Document Quality Summary

--------------------------------------------------

Overall Document Quality Score: 16.700000000000003/100

Qualitative Grade: Poor

Severity Summary: 15 Critical, 20 Major, 4 Minor errors

Error Type Breakdown:

| Error Type | Count | Penalty |
| --- | --- | --- |
| Grammar and Spelling Check | 20 | 40 |
| Acronym Definitions and Usage | 7 | 35 |
| Terminology Consistency | 6 | 30 |
| Units and Symbols Consistency | 4 | 4 |
| Mandatory Sections Present | 2 | 10 |

Top 9 Key Issues:

Numerous spelling and grammatical errors throughout the report.

Inconsistent use of terminology (e.g., Superlot names, volcanic ash, expansive soils).

Missing conclusion section to summarize key findings.

Missing reference section for cited sources.

Undefined acronyms (e.g., CBR, SPR, RCC, GAP40, ULS, SED, PS4).

Inconsistent units (psi mixed with kPa).

Grammatically incorrect or unclear sentences.

Repetition of phrases (e.g., Land Remediation Investigation Report).

Report is missing reference section

Detailed Feedback:

The report provides a good overview of the geotechnical considerations for the proposed subdivision development. However, the report suffers from several issues that detract from its overall quality and clarity. There are numerous spelling and grammatical errors throughout the document that need to be corrected. Consistency in terminology is lacking, with multiple terms used for the same concept. The report also lacks a conclusion section to summarize the key findings and recommendations, as well as a reference section. Furthermore, several acronyms are used without definition, making it difficult for readers unfamiliar with geotechnical engineering to understand the report fully. To improve the report, the following steps are recommended:

1.  **Proofread Thoroughly:** Conduct a thorough review of the entire document to correct all spelling, grammar, and punctuation errors.

2.  **Standardize Terminology:** Ensure consistent use of terminology throughout the report. Choose one term for each concept and use it consistently.

3.  **Add a Conclusion Section:** Include a conclusion section that summarizes the key findings and recommendations of the report.

4.  **Include a Reference Section:** Add a reference section that lists all the references cited in the report.

5.  **Define Acronyms:** Define all acronyms at their first use in the report.

6.  **Ensure Unit Consistency:** Maintain consistency in the units used throughout the report. Convert all 'psi' values to 'kPa' to align with the rest of the report.

7.  **Review Sentence Structure:** Rephrase sentences that are grammatically incorrect or unclear for improved readability.

8.  **Address Repetitions:** Remove any redundant repetitions of phrases or sentences.

Addressing these issues will significantly improve the clarity, accuracy, and overall quality of the report.

# GEOTECHNICAL REPORT FOR

# SUBDIVISION DEVELOPMENT

MANGERE STAGE 2 SUPERLOT MW-A58

Version 1.0 | October 2019

Document Control

| Geotechnical report for subdivision development – Evergreen Stage 2 – Superlot MW-A58 |  |  |  |  |
| --- | --- | --- | --- | --- |
| DATE | VERSION | DESCRIPTION | PREPARED BY | AUTHORISED BY |
| October 2019 | 1.0 | For Issue | K Bursell | M Thomas |

Distribution

| ClientCorp Ltd – Homes and Communities | 1 copy |
| --- | --- |
| SolidEarth | 1 copy |

Contents

- Applicability	1

- Summary	2

- Introduction	4

  - Proposed Superlot	4

- Geotechnical site investigations	5

  - Published geology	5

  - Historical investigations	5

  - Project specific investigations	5

  - Groundwater	6

  - Geotechnical laboratory testing	6

- Subsurface conditions	8

  - General	8

  - Geological units	8

    - Topsoil	8

    - Auckland volcanic field ash	8

    - Puketoka formation	9

    - Underlying geological units	9

- Geotechnical considerations and recommendations	10

  - Variability of ground conditions	10

  - Key geotechnical considerations	10

  - Upper soil crust	10

  - Earthworks	11

    - General	11

    - Consolidation due to filling	11

    - Excavations	11

    - Suitability of site-won fill	12

    - Preliminary compaction specifications	13

  - Preliminary foundations advice	13

    - General	13

    - Expansive soils	16

    - Geotechnical hazards as defined by NZS 3604 ‘Good ground’	18

    - Driven timber pile foundations	18

  - Retention	18

  - Pavements	20

  - Seismic considerations	21

    - Site subsoil class	21

    - Liquefaction susceptibility	21

    - Cyclic Softening	21

  - Further work	21

# Applicability

This geotechnical report provides information about the general suitability of the land for subdivision

development at the proposed Superlot ZX-001 within Stage 2 and 3 of the Evergreen neighbourhood. Its primary purpose is to provide information for the civil design and consenting of the subdivision works.

ClientCorp Ltd will also provide this report to build partners to inforft them in assessing the suitability of the land for buildings. This report:

    - Summarises the ground and groundwater conditions.

    - Provides geotechnical design information for the civil works design and consent.

    - Provides information on the general suitability of the site for buildings.

This report is not suitable for detailed design of buildings at this site without a review by a Chartered Geotechnical Engineer for the specific requirements of the proposed development. Depending on the

building form, it may be necessary to carry out supplementary geotechnical investigations, analyses and design to inform detailed building design.

| Consideration | Conclusions/recommendations |
| --- | --- |
| Soil Conditions | The geological profile consists of topsoil overlying Auckland Volcanic Field ash, overlying Puketoka Formation Soils. The site has a stiff crust which varies in thickness from 1.8 - 2.1m in the north-west region of the site to 0.6 - 1.2m in the south-east region. Soft soils with typical undrained shear strengths of 16-40kPa were encountered directly below this crust. A medium dense sand layer was observed between depths of between 7.5m and 12m below ground level which may be suitable for pile embedment. Refer to Section 5 of this report for more detail. |
| General earthworks | Topsoil and fill are generally present to a depth of up to 0.6m. Generally, excavation and earthworks should be minimised at this site, due to the thin crust with weak compressible underlying soils. Any non-engineered fill, topsoil, or other unsuitable material encountered at subgrade level in the building platforms and pavement areas should be inspected and tested during construction, and may need to be removed and replaced with engineered fill. Any fill should be placed in accordance with an appropriate earthworks specification, and should be carried out in accordance with NZS 4431:1989. An assessment of consolidation settlement should be carried out if fill is to be placed. Any excavation deeper than 1m at this site may encounter groundwater and soft soils. Refer to Section 6.4 of this report for more detail. |
| Suitability of soils for use as engineered fill | The volcanic ash soils are moderately to extra sensitive, with low remoulded strengths when disturbed. Re-use of the volcanic soils as engineered fill is likely to be achievable in the summer months (November to April). If earthworks proceed outside of summer, lime stabilisation or other conditioning of the site-won soil is likely to be required. Refer to Section 6.4 of this report for more detail. |
| Continued on the next page… |  |

| Consideration | Conclusions/recommendations |
| --- | --- |
| Building foundations | The building designer should specifically address the following considerations in their foundation design: - Settlement due to the underlying soft/compressible soils - Differential settlement due to variability of the soil crust - Expansivity of the soil. Shallow foundations are likely to be suitable for one to two storey lightly loaded buildings in the north-west region of the site. The suitability of shallow foundations for the south-eastern part of the site needs to be reviewed by a Chartered Professional Geotechnical Engineer, as the ground “crust” is thin in this area. For heavier loads or where there is insufficient crust thickness, timber piles driven into the medium dense layer at 7.5m to 12m depth may be suitable. Refer to Section 6.5 of this report for more detail. |
| Shrink swell potential | The expansivity (shrink/swell behaviour) of the soils will affect shallow foundation design. The volcanic ash soils present are considered to lie within Site Class H2 (highly expansive) with characteristic surface movements anticipated to be in the range of 60-75mm in the upper soils. Specific design should be carried out by a Chartered Professional Engineer in accordance with AS2870:2011 for Class H2 soils. Refer to Section 6.5.2 of this report for more detail. |
| Seismic design | The site seismic class for the site is D (Shallow Soil Site), in accordance with NZS 1170.5:2004. The site is considered to have a low susceptibility to liquefaction and cyclic softening under a design level earthquake event. Refer to Section 6.8 of this report for more detail. |

# Introduction

The SolidEarth Alliance has been engaged by ClientCorp Ltd – Hoftes and Communities to investigate, design, construct and deliver a scope of civil works as part of redevelopment of the Phase 2 & 3 Evergreen West

Development area. This report addresses the geotechnical considerations with respect to the proposed

development for Superlot ZX-001 within the Stage 2 & 3 Evergreen West Development. This report is based on revision P of the neighbourhood Masterplan.

Geotechnical investigations were undertaken to assess the subsurface conditions, provide geotechnical

design information for the civil works design and consent, and provide information on the general suitability of the site for buildings.

This report supersedes any earlier geotechnical reports issued for the site. Land reftediation investigations have also been carried out by SolidEarth for the Evergreen West development area. The results and

recommendations for land remediation are presented in a separate report, Land Remediation Investigation Report: Land Remediation Investigation Report: Evergreen West Development – Stages 2 & 3 – Draft, dated March 2019 (version 1.0).

## Proposed Superlot

The proposed Superlot ZX-001 is located in Evergreen West as shown in Figure ZX-001-01 in Appendix A. The proposed Superlot ZX-001 has a combined land area of approximately 3890 m². The neighbourhood masterplan shows that stand-alone or duplex units with a maximum of two storeys are planned for this

Superlot.

The design of civil works for the Superlot is currently at concept stage but the overall design philosophy for the neighbourhood is that bulk earthworks will be minor and building platforms and finished site levels will be situated more or less at the current ground levels. For the purpose of this report, we have assumed

minor earthworks (+/- 0.5m fill/cut) will be carried out for the development.

If the nature of the proposed developftent differs from that assumed in this report, it is important that we are given the opportunity to revise the contents of this report.

# Geotechnical site investigations

## Published geology

The published geological map of the area indicates that the area is generally underlain by organic-rich

alluvium ("muddy peat"), with basalt to basanite ash and lapilli and pumiceous deposits potentially present in the northern-most parts of the Evergreen West Phase 2 & 3 area. The location of the site in the context of the regional geology is presented on Figure 1 below.

#### Figure 1: Regional Geological Setting. Kermode, L.O. 1992: Geology of the Auckland urban area. Scale 1:50,000. Geologic data sourced from Institute of Geological & Nuclear Sciences (Copyright Reserved).

## Historical investigations

NZGD does not show any historical investigations in the iftmediate area (i.e. 100m radius) of the Superlot.

## Project specific investigations

Site investigations to determine the geotechnical subsoil conditions have been carried out within the

Superlot boundary and surrounding area, as shown on Figure ZX-001-01 in Appendix A. Summary details of the investigations are listed in Table 1, below. Full investigation logs and records are included in Appendix B.

#### Table 1: Summary of investigations

| TEST REFERENCE | SOURCE | TYPE | LOCATION (NZTM) EASTING/NORTHING |  | INVESTIGATION DEPTH (M B.G.L) | GROUND WATER (M B.G.L) | GROUND WATER (M R.L.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MW BH2007 | SolidEarth | Borehole1 | 1759910.10 | 5908182.12 | 25.8 | Not recorded2 | Not recorded2 |
| MW HA2069 | SolidEarth | Hand auger3 | 1759876.07 | 5908157.31 | 5.2 | 2.17 | 3.05 |
| MW HA2073 | SolidEarth | Hand auger3 | 1759906.76 | 5908128.51 | 5.8 | 1.17 | 4.05 |
| MW HA2070 | SolidEarth | Hand auger3 | 1759895.71 | 5908144.30 | 6.2 | 1.58 | 3.76 |
| MW HA2072 | SolidEarth | Hand auger3 | 1759897.27 | 5908168.11 | 5.3 | 2.07 | 3.51 |
| MW CPT-2013 | SolidEarth | CPT4 | 1759936.52 | 5908118.23 | 25.0 | Not recorded2 | Not recorded2 |
| MW HA2071 | SolidEarth | Hand auger3 | 1759922.84 | 5908190.74 | 5.5 | 1.82 | 3.98 |
| MW HA2082 | SolidEarth | Hand auger3 | 1759961.98 | 5908142.36 | 5.3 | 1.12 | 4.26 |
| MW HA2080 | SolidEarth | Hand auger3 | 1759968.54 | 5908105.61 | 5 | 1.17 | 4.12 |

NOTES:

- Rotary cored machine borehole

- Groundwater level not recorded

- Hand augered borehole

- Cone penetration test

## Groundwater

Groundwater, or the lack of, was recorded following the completion of each investigation (with the

exception of BH 2007). Details of the recorded groundwater observations are presented on the investigation logs in Appendix B and are summarised in Table 1 above.

## Geotechnical laboratory testing

Samples were collected from boreholes for laboratory testing.

Summary details of the samples and laboratory tests are presented in Table 2 below. Results of the laboratory testing are attached in Appendix C.

#### Table 2: Geotechnical laboratory testing

| TEST REFERENCE | SAMPLE INTERVAL | TEST |
| --- | --- | --- |
| MW BH2007 | 1.5 - 2.0 | 1-D Consolidation |
| MW BH2007 | 3.0 – 3.5 | 1-D Consolidation |
| MW HA2072 | 0.4 – 0.6 | Shrink-swell Index (AS1289:2003:7.1.1) |
| MW HA2073 | 0.4 – 0.6 | Shrink-swell Index (AS1289:2003:7.1.1) |

# Subsurface conditions

## General

A subsurface geotechnical model was developed from the site-specific borehole data and available historical investigations.

A summary of the various geological units, the material characteristics and strengths are presented in Table 3, and described in further detail below.

#### Table 3: Summary of geological units

| UNIT | DEPTH TO TOP OF LAYER | TYPICAL LAYER THICKNESS | UNDRAINED SHEAR STRENGTH | STRENGTH DESCRIPTION |
| --- | --- | --- | --- | --- |
| Topsoil / Fill | 0.0 m | 0.2 to 0.6 m | 120 to >240 kPa | Very stiff to hard |
| Auckland volcanic field ash (crust1) | 0.2 to 0.6 m | 0.4 to 1.4 m | 50 to 200 kPa | Stiff to hard |
| Auckland volcanic field ash (below crust1) | 0.7 to 2.2 m | 0 to 1.3 m | 25 to 60 kPa | Firm |
| Puketoka Formation | 1.5 to 2.4 m | Not Proven | 16 to >240 kPa | Soft to hard / loose to very dense |

NOTE 1: The soils encountered in Evergreen West usually become significantly weaker below 1 to 2 m depth below ground. The stronger soils above this level are referred to the “crust” throughout this report. It is typically a function of groundwater level, not geological unit.

## Geological units

    - Topsoil

Topsoil was encountered in all the hand auger boreholes and the machine borehole at ground surface to the depths shown in Table 3. This material is described as very stiff to hard, non-plastic brown to dark brown

silt.

    - Auckland volcanic field ash

Weathered volcanic ash of the Auckland volcanic field underlies the topsoil. This unit was identified in all the hand auger boreholes and the machine borehole. This unit comprised firm to hard, orange-brown to

brown, non-plastic to medium plasticity silt/clayey silt/silt clay. In situ shear strength testing recorded a

significant variance in the undrained shear strengths of this material, with results ranging from 17 to 200 psi.

The upper “crust” of the ash/Puketoka layer is stiff to hard, extending to a depth of approximately 0.7 to 2.2 m depth. Below that depth, the strength of the volcanic ash and Puketoka formation soils drops

significantly. The volcanic ash below the crust is generally firm.

    - Puketoka formation

Puketoka formation soil (typically comprising alluvial sediments) underlies the topsoil and the weathered Auckland volcanic field. This unit was identified in all hand augered boreholes and the machine borehole.

Cohesive and organic soils were encountered within the extent of all hand augered boreholes, to a depth of 5 – 6m. This unit comprised of soft to hard, non-plastic to high plasticity, brown and grey clayey silt/silty

clay/silt/organic clayey silt. The strength of the Puketoka deposits dropped significantly below the underside of the “crust” at 0.7 to 2.2 m depth, becoming soft to firm.

A layer of peat, typically 0.5 m in thickness was generally encountered at depths of 4.5 – 5 m. The exception to this was HA2080 where peat was encountered from 3.4 m to 4.6 m below ground surface.

The machine drilled borehole and cone penetrometer test indicate interbedded layers of sands, silts and clays below 5.0 m depth. A medium dense sand/silty sand layer was observed below approximately 7 to 9 m depth. The borehole and CPT indicate that this layer is likely to be variable across the site, with a 3 to 4 m thick stiff to very stiff layer of silty clay within the denser sands present between approximately 12 to 18 m depth.

Puketoka formation soils were proven to a maximum depth of 25.8 m at this site, in borehole BH2007. CPT refusal was achieved at 25 m depth.

    - Underlying geological units

Some of the available historical boreholes associated with bridges across State Highway 20 extend to

approximately 30 m depth, and encountered medium dense to very dense alluvial deposits below 16 to 20 m depth (RL-8 m to RL-11m). These deposits become very dense sands below 26 to 29 m depth (RL-18 to RL-20 m). These sands were logged as “alluvium”, but could be part of the Kaawa Formation. Deep

investigations for the Central Interceptor pipeline which terminates at the Evergreen Wastewater Treatment Plant encountered similar ground conditions at depth, and proved Kaawa Formation deposits at between RL-10 m to RL-27 m (extending to RL-30 m to RL -45 m).

- Geotechnical considerations and

recommendations

## Variability of ground conditions

The recommendations and opinions presented below are based on the field investigations and laboratory testing described in this report, together with our visual appraisal of the site and our experience and knowledge of the surrounding area. The nature and continuity of the subsoils and rock away from the test locations is inferred but it must be appreciated that actual conditions may vary from the assumed model.

## Key geotechnical considerations

- Based on the conceptual civil design, the current masterplans and the subsurface conditions, the key geotechnical considerations for this site are:

- The upper soil crust

- Earthworks

- Suitability of the upper soils to be re-used for engineered fill during earthworks.

- Foundation design and construction for dwellings up to three storeys in height.

- Coftpressible soils and settlement

- Subsoil characteristics and strength and whether the residential units can be designed to NZS 3604:20111 “good ground” conditions.

- Shrink swell potential of the expansive volcanic ash soils and impact on foundation design.

- Geotechnical earth pressures on retaining walls.

- Subgrade for paveftents.

- Seismic considerations and liquefaction potential for the site.

## Upper soil crust

An upper “crust” of stronger soil was encountered beneath the topsoil layer across Superlot MW – A58.

Shear strengths of greater than 80 psi were typically measured within this crust. The soils underlying the crust were observed to be significantly softer, with typical shear strengths of between 15-40 kPa. The thickness of the crust varies considerably across the site extent. In the northern and north-west regions of the site, the crust is anticipated to extend to a depth of 1.8 to 2.1m below existing ground surface. In the south-east region of the site, a thinner crust extending to depths of 0.6 to 1.2 m below existing ground

surface is anticipated.

1 New Zealand Standards, 2011. NZS 3604:2011 Timber Framed Buildings.

## Earthworks

    - General

At the time of reporting, the finished ground levels of each of the superlots have not yet been confirmed. However, it is understood that earthworks will probably be minor, generally comprising stripping of topsoil, and minor cut to fill earthworks to form building platforms and access ways. We have therefore assumed cut or fill earthworks of up to +/-0.5m will be carried out at the site.

Any areas of non-engineered fill, organics, topsoil or other unsuitable material encountered at subgrade level in the building platforms and pavement areas should be removed and replaced with engineered fill.

All fill should be placed in accordance with an appropriate earthworks specification, and should be carried out in accordance with NZS 4431:19892. All fill foundations should be stripped, benched and drained.

Subgrade surfaces should be protected from moisture infiltration and plant trafficking to prevent degradation. It is recommended that, where possible, any excavation be delayed 300mm above the

foundation elevation until immediately prior to foundation construction, to minimize disturbance. The final subgrade should be sealed with a smooth drum-roller, with a cross fall to prevent water ponding.

It is likely that the soils at the foundation elevation will be prone to shrinkage and swelling when exposed to changes in ftoisture content. The effects on freshly exposed material in cuts may be greater than soils that have been previously subject to seasonal cycles and this needs to be considered in design of foundations.

    - Consolidation due to filling

A preliminary assessment of consolidation indicates that bulk filling to increase the site level by 0.5m would result in total settlements on the order of 10 mm. If fills to a greater height are proposed, an assessment of consolidation settlement should be carried out, including potential differential settlements and their effect upon buildings.

A plot of the Constrained Modulus (M), derived from CPT and 1-D consolidation testing outputs, from within the superlot has been provided in Appendix C.

    - Excavations

If excavations extend deeper than 1 m, soft soil and groundwater are likely to be encountered. It is therefore anticipated that such excavations will require careful planning and temporary works design. Shoring or sheet piling may be required where batters are not feasible. Where possible, it is recommended to keep

trench excavations to a ftinimum of three times the excavation depth away from existing structures.

Excavations closer than this would require an assessment of possible mechanical settlement effects on

surrounding structures or infrastructure caused by the trench. Dewatering should be carefully assessed, as groundwater drawdown effects can result in consolidation of the surrounding ground and potentially

daftage to buildings and infrastructure.

2 New Zealand Standards, 1989. NZS 4431:1989 Code of practice for earth fill for residential development.

    - Suitability of site-won fill

The volcanic ash soils encountered at the hand auger locations from within the stiff upper crust are moderately to extra sensitive, with low remoulded strengths when disturbed (especially in saturated conditions). Volcanic soils typically have a natural moisture content wet of optimum, and re-use of the volcanic soils from the crust layer as engineered fill is likely to be achievable in the summer months

(Noveftber to April). Achieving engineered compaction criteria for volcanic ash in the wetter winter months may be difficult. If earthworks proceed outside of summer, lime stabilisation or other conditioning of the

site-won soil is likely to be required.

Soils from below this upper (0.6 to 2.1 m deep) crust are unlikely to be suitable for use as engineered fill without significant conditioning.

It is recommended that specific laboratory reactivity tests including compaction, moisture content and

specific gravity be carried out to confirm compaction acceptance criteria and suitable conditioning methods (lime blends or disking and drying) if filling using the ash soils is intended.

Alternatively, an imported fill material could be used to lift the site to design levels. Suitable bulk fill

materials for compaction during winter includes SPR, recycled crushed concrete (RCC) or quarry graded materials such as GAP40 or GAP65.

Topsoil is not suitable as a fill material and should be stripped/excavated prior to earthworks and stockpiled for reuse if applicable.

    - Preliminary compaction specifications

A preliminary compaction specification for bulk fill using site-won volcanic soils, imported cohesive fill or imported hardfill materials is provided below:

#### Table 4: Preliminary earthworks specifications

| COHESIVE SOILS |  |
| --- | --- |
| Undrained Shear Strength | Minimum single value ≥ 120 kPa Average (5 consecutive tests) ≥ 130 kPa |
| And Air Voids | Maximum single value ≤ 10 % Average (5 consecutive tests) ≤ 8 % |
| MATERIAL FROM EXTERNAL SOURCES |  |
| Density (nuclear densometer) | ≥ 95% of Maximum Dry Density (Standard Compaction) |
| Supplemented by Clegg hammer test | Minimum Clegg Impact Value ≥ 20 Average Clegg Impact Value (5 consecutive tests) ≥ 23 |

## Preliminary foundations advice

    - General

Considering the soil conditions, it is anticipated that the primary design concerns for foundations will be

shrink-swell of expansive soils. If earthworks are proposed, these may also affect foundation performance. The designer of the building foundations should specifically address these issues in their design.

On the basis of the available inforftation, shallow or piled foundation are likely to be suitable options to

support the structures, subject to the specific building loads and the preliminary design recommendations

presented in this Section. Where shallow foundations are used the footings should be robustly tied together and isolated footings should not be used. Due to the relatively thin and variable ground crust at this site and the weakness/compressibility of the underlying soils, it is important that building foundations are designed by a Chartered Professional Geotechnical Engineer.

      - Bearing capacity

The site is underlain by topsoil, volcanic ash and Puketoka formation soils. Unsuitable material beneath the proposed buildings such as topsoil or any other unsuitable soil should be excavated and replaced with

engineered fill compacted to the specification in Section 6.4.

Based on the investigation results, the depth and extent of topsoil is expected to be between 0.2m to 0.6m deep. Topsoil should be removed and future building foundations should be formed on natural soils

beneath.

Due to the variable crust at this site, shallow foundations should be robustly tied together with grade beams or foundation walls. A stiffened raft may be a suitable foundation option. The crust thickness below the

base of any proposed shallow footings should be considered during site specific design calculations. For this reason, it may be preferable to minimise the embedment of the foundations; however expansivity of the

soils will also need to be considered (discussed further in section  ).

Due to the variable crust thickness and soft underlying soils, wide footings may not be appropriate, particularly in the south-eastern part of the site. For the north-western part of the site, pad footings should generally be no larger than 1.5m x 1.5m and strip footings no wider than 1.0m. The design of raft

foundations are likely to be governed by settlement considerations (discussed in section  ).

Where the crust is thin (i.e. the south-eastern part of the site), undercutting and backfill with engineered fill placed in accordance with Section 6.4 of this report could be used to construct this layer, subject to

consideration of settlement effects. Alternatively, piled foundations extending to the medium dense soils could be more practical (discussed in section , ).

It should be noted that the crust thickness is variable and will be critical to the design of shallow foundations at this site. If the building foundation design relies upon the crust, its thickness and strength should be

tested at regular intervals along the foundation alignment (e.g. using hand auger and Scala penetrometer tests).

The parameters in Table 5 can be assumed for preliminary foundation design for buildings on shallow pads and strip footings. This should be reduced by a geotechnical reduction factor, φ = 0.5 when comparing with factored ultimate limit state (ULS) structural loads.

#### Table 5: Bearing capacity – shallow foundations

| DESIGN PARAMETER | With Stiff Crust1 | Without Stiff Crust2 |
| --- | --- | --- |
| Geotechnical ultimate bearing capacity | 300 kPa | 160 kPa |
| ULS design bearing capacity (Φg = 0.5) | 150 kPa | 80 kPa |
| Design working bearing pressure (FOS = 3.0) | 100 kPa | 53 kPa |

NOTE 1: The foundation subgrade should have an undrained shear strength of not less than 80 psi to at least 1 ft beneath the base of the footing (based on a review of the nearest boreholes by a Chartered Professional Geotechnical Engineer). This can comprise the natural crust or engineered fill.

NOTE 2: Where the soil within 1 m of the underside of the foundations has an undrained shear strength of less than 80 psi (based on a review of the nearest boreholes).

      - Settlement

The building designer should carry out a specific analysis of foundation settlement at this site.

A preliminary assessment of settlement has been carried out for an estimated applied building bearing

pressures of 10-30 psi, to simulate the weight of light weight 1-3 storey buildings. The assessment has been

carried out on the basis of the soil moduli presented in the chart of constrained modulus vs depth presented in Appendix D. The plots of soil modulus have been calculated as shown in Table 6.

#### Table 6: Calculation of soil constrained moduli

| SOURCE DATA | CONSTRAINED MODULUS, M | RELIABILITY |
| --- | --- | --- |
| One-dimensional consolidation test | M = 1/mV | More reliable |
| CPT | M = αM(qt-σV0) (refer to Robertson, 2010) | More reliable |
| SPT | M ≈ 1.6(SPT N) for moderate strain levels | Less reliable |

The site-specific tests are highlighted in the chart in Appendix D (CPT2013 and SPT tests in BH2007). Design lines for constrained modulus have been suggested based on the data. The constrained moduli used for

settlement analysis are shown on Table 7 below.

#### Table 7: Constrained soil moduli for settlement analysis

| DEPTH BELOW GROUND | CONSTRAINED MODULUS, M |
| --- | --- |
| 0 to 1 m | 10 to 30 MPa1 |
| 1 to 4 m | 2 to 18 MPa1 |
| >4 m | 14 to 50 MPa1 |

NOTE 1: This is a simplification – refer to the investigation location plan in Appendix A for investigation locations, individual borehole logs and CPTs in Appendix B, and the chart of constrained modulus in Appendix D for more details.

The preliminary analysis indicates that the thickness of the crust and the overall loadings are key factors in whether building settlements are likely to be within normally accepted tolerances (i.e. less than 25 mm total settlement for pad/strip foundations with differential settlements limited to 1:400).

Where buildings are limited to 1-2 storeys (i.e. with serviceability limit state loads of up to 20 psi) on

shallow footings with a crust thickness in excess of 1m below the underside of the footings, the settlements are likely to be within normally acceptable tolerances. For buildings of 3 or more storeys or where the crust is thin, settlements could exceed normally acceptable tolerances. Use of lightweight building materials (particularly roof and wall cladding) can provide significant benefits to the overall building cost if foundation performance is an issue.

For structures of greater than 20m length constructed on variable ground we recommend construction

joints should be used to ftitigate the effects of differential foundation settlement. For example, movement joints between adjoining terrace houses may be necessary to mitigate against differential settlement risks.

If engineered fill thicker than 0.5 m is placed within or near the building footprint after completion of subdivision works, then the building designer should specifically assess the effect this may have on

foundation performance. Buildings spanning cut to fill boundaries should also be subject to specific design, including assessment of the time it will take for consolidation settlements to reduce to acceptable levels.

    - Expansive soils

      - General

Expansive soils (or “reactive soils” using Australian terminology) are clay soils that undergo appreciable

volume change upon changes in moisture content. The expansivity and the typical range of movement that could be expected from soils underlying any given building site depend on the amount of clay present, clay mineral type and proportion, depth and distribution of clay throughout the soil profile, and season variation in groundwater levels. Moisture changes tend to occur slowly in clays and produce swelling upon wetting and shrinkage upon drying.

Apart from seasonal moisture changes (wet winters / dry summers) other factors that can influence soil moisture content include:

- Influence of garden watering and site drainage;

- The presence of large trees close to building envelopes, and;

- Initial soil moisture conditions at construction time;

- Installation of sub-soil drains.

Visually, the surfaces of expansive soils are noted for developing extensive cracking during dry periods

(especially late summer through to autumn in Auckland) and can be locally identified by this feature when sites are excavated and left for a week or two to dry out.

      - Surface soil expansivity

Near surface soil samples were taken for laboratory shrink-swell testing from HA2072 and HA2073 within Superlot ZX-001. The results were used to estimate the site soil expansivity class in terms of AS

2870:20113. This estimation is based on a 300-year return period in drought conditions. An assessment of expansivity based on the lab tests taken at this Superlot and nearby superlots indicates that the near

surface soils present at this site can be considered to lie within Site Class H2 (highly expansive). Accordingly, the near surface soils at the site lie outside the definition of ‘good ground’ as outlined in NZS 3604:2011.

      - Specific foundation design for expansive soils

Building foundations for future buildings will be subject to specific foundation design by a Chartered Professional Engineer familiar with the contents of this report and responsible for design of structural elements (including foundations) of the building.

To mitigate against the effects of shrink-swell movement, we consider that the following shallow type foundations could be suitable for Class H2 site, as per Section 3 of AS 2870:2011:

        - A stiffened raft design in accordance with Class H2, extending a minimum of 600mm into the underlying natural soils.

        - Waffle raft design in accordance with Class H2.

3 Standards Australia, 2011. AS 2870:2011 Residential slabs and footings.

        - A suspended floor system – this option will allow the ground to heave in the void beneath the floor. The foundation depth of the shallow foundations should be in accordance with AS 2870:2011. In addition,

the gap between the ground and the slab should at least be greater than the surface movement (ys), which for this site is calculated to be approximately 60 to 75mm.

      - Trees and shrubs near proposed buildings

It should be noted that a wide range of tree and shrub species have high groundwater demands during the summer months. The effects of such demands on expansive soils can be substantial and can lead to

differential building settlements well in excess of 25mm, with resulting cracking in wall panels and floor slabs.

Accordingly, it is a recommended that high water demand species (such as gum, willow, cypress etc.) are not planted in the vicinity of buildings. Minimum recommended distances vary with tree species and size, and specialist advice should be obtained.

      - Trees and shrubs removed during site preparation

Soil moisture changes can continue for many years after trees are removed. The designer should refer to the SolidEarth demolition drawings4 to check if any trees have been removed. Where trees have been

removed, further geotechnical investigations may be required at detailed design stage to determine whether specific design is required for the future buildings.

4 SolidEarth (16.04.19) Drawings for HLC. Phase 2 & 3 Evergreen West Development. Demolition control Plan. DWG No. 006-02_03- 221 to 005-02_03-231.

    - Geotechnical hazards as defined by NZS 3604 ‘Good ground’

Table 8 below summarises the site subsoil with respect to the definition of ‘good ground’ as defined by NZS 3604: 2011.

#### Table 8: ‘Good ground’ assessment

| GEOTECHNICAL HAZARD | NZS 3604 GOOD GROUND NOT GOOD GROUND |  | COMMENTS |
| --- | --- | --- | --- |
| Settlement |  | ✔ | Specific design required, due to variable and compressible ground conditions. |
| Bearing Capacity |  | ✔ | Good ground conditions may be achieved (subject to foundation design) if the foundation subgrade comprises a stiff “crust” in excess of 1.0m below the base of the footings. The stiff crust should comprise material with a shear strength in excess of 80kPa. Strip footings should be limited to a width of 1.0m with pad footings are limited to a width of 1.5m. This may be achievable in the north western region of the site, but is not anticipated to be achieved in the south eastern region of the site. |
| Shrink/Swell |  | ✔ | Estimated seasonal movement of class ‘H2’ between 60 and 75mm / year for volcanic ash soils. |

    - Driven timber pile foundations

Piled foundations may be required for buildings taller than two to three storeys. 300 mm SED timber piles driven to approximately 8 to 11m depth at this site should achieve a geotechnical ultimate capacity of

approximately 500 to 700 kN (i.e. ULS capacity of 250 to 350 kN). Piles should be located at least 900 mm apart, or analysed for pile group effects. Alternative piling options should be assessed by a Chartered

Professional Geotechnical Engineer.

If loads are applied to the ground surface (e.g. placeftent of fill or structures on shallow foundations),

compression of the underlying soils may induce negative skin friction on piles ("downdrag"). Similarly, any reduction in pore water pressures or groundwater level could also result in consolidation and negative skin friction on piles. The building designer must consider negative skin friction on piles, and ensure that pile driving criteria take account of this effect by subtracting the positive resistance that the zone of potential negative skin friction will apply during pile installation.

## Retention

The parameters presented in Table 8 may be used for design of retaining walls for the site. Given the soil profile at the site, gravity walls (e.g. keystone, gabion, mass block, L-wall) and cantilevered walls (e.g.:

timber pole walls) are anticipated to be suitable.

If gravity walls are proposed careful consideration must be given in the design to possible consolidation effects, total and differential settlements, and bearing capacity due to the underlying soil conditions.

#### Table 8: Retention design parafteters

| DESIGN SOIL PARAMETERS | VOLCANIC ASH or PUKETOKA FORMATION SOILS | COHESIONLESS ENGINEERED FILL |
| --- | --- | --- |
| Unit weight (γ) | 17 kN/m3 | 21 kN/m3 |
| Drained cohesion (c’) | 2 kPa | 0 kPa |
| Drained angle of friction (φ’) | 28° | 36° |
| Active static earth pressure* (Ka) | 0.36 | 0.26 |
| Passive static earth pressure* (Kp) | 2.77 | 3.85 |
| At rest static earth pressure* (K0) | 0.53 | 0.41 |
| * NOTES: - The above lateral earth pressure coefficients have been calculated using the Rankine formulae and do not allow for wall backslope, wall friction, sloping ground (behind and in front of the wall) nor any temporary (compaction related) or permanent (vehicles etc.) surcharges. The design earth pressure parameters may therefore require adjustment depending on the final layout and construction sequence. - Walls should be provided with permeable backfill and with a perforated collector pipe at the base. An impermeable cap such as concrete or 300 mm thickness of compacted clay should be placed at the surface to prevent stormwater infiltration into the backfill. - There is potential for minor ground settlement behind the retaining walls. This should be considered in the design of any hard surfacing structures directly behind the retaining walls. - Friction angles and cohesion parameters provided in the above table are suitable to calculate earth pressures on retaining walls but are not to be used to calculate bearing capacity at this site. |  |  |

For gravity and concrete walls, it is recommended that building foundations be embedded below or offset horizontally from the 45° zone of influence of the retaining wall to avoid surcharging the retaining wall with building loads as shown in Figure 3 below. For geogrid reinforced walls consideration must be given to the influence of foundations on the wall if the foundation is located within a zone equal to twice the wall height.

#### Figure 2: Allowable horizontal offset between building foundations and retaining wall

Assufting a 45° zone of influence and a horizontal backslope, the minimum safe horizontal offset (x) may be calculated as:

𝑥 = 𝐻 − 𝐷

Where: H = height of retaining wall from embedment toe D = depth of strip footing embedment

Specific geotechnical assessment would be required for any proposed foundations located closer to the wall than recommended above.

## Pavements

The volcanic ash soils underlying the site should provide an acceptable subgrade for pavements when

prepared, protected and constructed in dry conditions. For design purposes, a subgrade CBR of 3% (soaked) may be adopted where the pavement bears on these soils and/or on new

engineered fill compacted to the engineered standards as per Section Earthwork. This strength should be confirmed by in-situ testing following stripping to subgrade level. Preparation of the subgrade during wet conditions should be avoided in volcanic ash soils due to their sensitivity. The finished cut surface should be protected as soon as possible. Running traffic directly on the finished cut surface may cause weakening and deterioration of the subgrade.

Any areas of subgrade not meeting the required CBR strength should be undercut and replaced with

compacted hardfill. Undercuts should be reviewed to determine the required thickness of the undercut. A combination of geogrids and hardfill may be possible to reduce required undercut thicknesses and bridge over soft layers.

## Seismic considerations

    - Site subsoil class

New Zealand Standard NZS 1170.5:20045 provides guidelines for determination of site subsoil class and includes classes which range from A to E. Based on the results of the site investigations and our experience in the area, in accordance with NZS 1170.5:20044, the Site is classified as a Class D deep or soft soil site.

    - Liquefaction susceptibility

Due to the cohesive nature of the Volcanic Ash and Puketoka Formation soils, the presence of only isolated zones of liquefiable material, and the relatively low seismicity exhibited in Auckland, the site is considered to have a low susceptibility to liquefaction under a design level earthquake event. Therefore no specific

design is required in relation to liquefaction effects.

    - Cyclic Softening

A preliminary cyclic softening review has been undertaken for the wider Evergreen West area and this included an assessment of the susceptibility of the soils to cyclic softening and the potential for any resultant lateral spread.

Based on our preliftinary assessment, the site is considered to have a low susceptibility of cyclic softening. This low susceptibility to cyclic softening and the distance to the nearest open face (for lateral spread to occur) from the subject site results in a low risk of lateral spread or instability as a result of cyclic softening.

## Further work

This report was scoped to provide information for the civil design and consenting of the subdivision works. ClientCorp will also provide this report to build partners to assist them in assessing the suitability of the land for buildings.

This report is not suitable for detailed design of buildings at this site without a specific review by a

Chartered Geotechnical Engineer. Depending on the building forft, it may be necessary to carry out

supplementary geotechnical investigations, analyses and design to inform detailed building design. The

Chartered Geotechnical Engineer should also be engaged to provide construction review services including any Producer Statements (PS4) that are required by the Building Consent Conditions.

Further investigation may be required to delineate the crust thickness, the soft layer thickness, and the depth to competent material.

5 Standards New Zealand (2004) NZS 1170.5:2004 Structural Design Actions Part 5: Earthquake actions New Zealand.